# CITY OF HUMBOLDT

# BYLAW NO. 01/2026

The Building Placement and Elevation Bylaw

The Council of the City of Humboldt, in the Province of Saskatchewan, enacts as follows:

Short Title

- This Bylaw may be cited as The Building Placement and Elevation Bylaw, 2026.

Definitions

- In this Bylaw:

- “adverse effect” means actual or potential damage or harm to property, the environment or the safety, health or welfare of people.

- “lot grading plan” means an engineered plan demonstrating the type of drainage for each property in a developed area submitted with detailed subdivision plans and approved by the City of Humboldt prior to subdivision construction.

- “professional surveyor” means a surveyor registered with the Saskatchewan Land Surveyors Association.

- “property” means a legal parcel/lot registered with the Information Services Corporation.

- “real property report” means a certificate provided by a professional surveyor that verifies the elevations and location of a building.

- “site drainage plan” means a plan submitted with the building permit process to identify the type and direction of storm drainage on a lot.

Purpose

- The purpose of this Bylaw is to regulate the placement and elevation of buildings on private properties to minimize adverse effects.

Street and Lot Grades

- Drainage Plans

- Every applicant for a permit to construct any residential dwelling shall provide a site drainage plan for the property. The site drainage plan must comply with the parameters found within the lot grading plan for the subdivision where a lot grading plan is available.

- In the case of any property located in an area without an approved lot grading plan, a site drainage plan must be prepared by a professional surveyor and approved by the Department of Public Works.

Building Elevations and Locations

- Prior to excavation of the building foundation, a professional surveyor must be engaged to:

- Mark the location of property boundaries.

- Mark the location of the building foundation to the extent necessary to ensure proper placement of the building within the site; and

- Mark the approved building elevation to the extent necessary to ensure the completed building complies with the site drainage plan.

Real Property Report

- Following the completed construction of the bottom slab or basement, and prior to requesting an occupancy permit, the applicant shall obtain a Real Property Report from a professional surveyor. The Real Property Report shall include:

- The location of the building, in relation to the property lines

- The elevation of the top of footing

- The elevation of the project benchmark

_________________________________

Mayor – Rob Muench

_________________________________

City Clerk – Lori Yaworski

INTRODUCED AND READ A FIRST TIME THIS 23rd DAY OF FEBRUARY 2026.

READ A SECOND TIME THIS 23rd DAY OF FEBRUARY 2026.

READ A THIRD AND FINAL TIME THIS 23rd DAY OF FEBRUARY 2026.